PART NO.: CHIP RESISTORS                     FILE NO.:

1.  INSTRUCTION:

THIS SHEET IS THE STATEMENT OF THE THICK FILM CHIP RESISTORS SPECIFICATION THAT ROYALOHM’S PRODUCTIONS CAN MEET.

2.  RATING:

| TYPE |  | 0402 | 0603 | 0805 | 1206 | 1210 | 2010 | 2512 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| POWER RATING |  | 1/16W | 1/10W | 1/8W | 1/4W | 1/3W | 3/4W | 1W |
| MAX.WORKING VOLTAGE |  | 50V | 50V | 150V | 200V | 200V | 200V | 200V |
| 100VMAX. OVERLOAD VOLTAGE |  | 100V | 100V | 300V | 400V | 400V | 400V | 400V |
| DIELECTRIC WITHSTANDING VOLTAGE |  | 0402：100V；0603：300V；other：500V |  |  |  |  |  |  |
| RESISTANCE RANGE | ±1% | 0402：100--1M；other 10--1M |  |  |  |  |  |  |
|  | ±5% | 0402：100--1M；other 1--10M |  |  |  |  |  |  |
| TOLERANCE |  | ±1%, ±5% |  |  |  |  |  |  |
| RATED AMBIENT TEMP. |  | 70℃ |  |  |  |  |  |  |
| TEMP.RANGE |  | -55℃ --- +125℃ |  |  |  |  |  |  |

2.1 POWER RATING:

RESISTORS SHALL HAVE A POWER RATING BASED ON CONTINUOUS LOAD OPERATION AT AN AMBIENT TEMPERATURE OF 700C. FOR TEMPERATURE IN EXCESS OF 700C, THE LOAD SHALL BE DERATE AS SHOWN IN FIGURE 1.

#### FIGURE 1

2.2 VOLTAGE RATING:

RESISTORS SHALL HAVE A RATED DIRECT-CURRENT (DC) CONTINUOUS WORKING

VOLTAGE OR AN APPROXIMATE SINE-WAVE ROOT-MEAN-SQUARE (RMS) ALTERNATING-CURRENT (AC) CONTINUOUS WORKING VOLTAGE AT COMMERCIAL-LINE FREQUENCY AND WAVEFORM CORRESPONDING TO THE POWER RATING, AS DETERMINED FROM THE FOLLOWING FORMULA:

RCWV =√  P × R

WHERE: RCWV = RATED DC OR RMS AC CONTINUOUS WORKING VOLTAGE AT

COMMERCIAL-LINE FREQUENCY AND WAVEFORM (VOLT.)

P = POWER RATING (WATT.)

R = NOMINAL RESISTANCE (OHM)

IN NO CASE SHALL THE RATED DC OR RMS AC CONTINUOUS WORKING VOLTAGE BE GREATER THAN THE APPLICABLE MAXIMUM VALUE.

THICK FILM CHIP RESISTORS

3.  DIMENSION FOR CHIP:

DIMENSION: mm

| TYPE | L | W | H | l1 | l2 |
| --- | --- | --- | --- | --- | --- |
| 0402 | 1.00  0.10 | + 0.05 0.50 - 0.05 | 0.35  0.05 | 0.20  0.10 | 0.25  0.10 |
| 0603 | 1.60  0.10 | + 0.15 0.80 - 0.10 | 0.45  0.10 | 0.30  0.20 | 0.30  0.20 |
| 0805 | 2.000.15 | +0.15 1.25 -0.10 | 0.550.10 | 0.400.20 | 0.400.20 |
| 1206 | 3.100.15 | +0.15 1.55 -0.10 | 0.550.10 | 0.450.20 | 0.450.20 |
| 1210 | 3.100.15 | +0.15 2.60 -0.10 | 0.550.10 | 0.500.25 | 0.500.20 |
| 2010 | 5.000.15 | +0.15 2.50 -0.10 | 0.550.10 | 0.600.25 | 0.500.20 |
| 2512 | 6.350.15 | +0.15 3.20 -0.10 | 0.550.10 | 0.600.20 | 0.500.20 |

4.  STRUCTURE:

1: HIGH PURITY ALUMINA SUBSTRATE

( 96% AL2O3、0.3±0.1%CaO 、1.0±0.3%MgO 、2..1±0.05%SiO2  )

2,3: PROTECTIVE COVERING

4: RESISTIVE COVERING

5: TERMINATION (INNER) Ag/Pd

6: TERMINATION (BETWEEN) Ni PLATING

7: TERMINATION (OUTER) Sn PLATING

THICK FILM CHIP RESISTORS

- MARKING:

(1) FOR 0402 SIZE. DUE TO THE VERY SMALL SIZE OF THE RESISTOR’S BODY, THERE IS NO MARKING ON THE BODY.

(2) 5% TOLERANCE: THE FIRST TWO DIGITS ARE SIGNIFICANT FIGUTRES OF RESISTANCE AND THE THIRD DENOTES NUMBER OF ZEROS FOLLOWING

EXAMPLE:

33000  33K

(3) 5% TOLERANCE: BELOW 10 SHOW AS FOLLOWING

EXAMPLE:

2.2

(4) 1% TOLERANCE: 4 DIGITS, FIRST THREE DIGITS ARE SIGNIFICANT, FORTH DIGIT IS NUMBER OF ZEROS. LETTER R IS DECIMAL POINT.

EXAMPLE:

2700  2.7K

10.5

(5) STANDARD E-96 SERIES VALUES (±1% TOLERANCE) OF 0603 SIZE. DUE TH THE SMALL SIZE OF THE RESISTOR’S BODY, 3 DIGITS MARKING WILL BE USED TO INDICATE THE ACCUATE RESISTANCE VAUE BY USING THE FOLLOWING MULTIPLIER & RESISTANCE CODE.

MULTIPLIER CODE:

| CODE | A | B | C | D | E | F | G | H | X | Y | Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MULTIPLIER | 100 | 101 | 102 | 103 | 104 | 105 | 106 | 107 | 10-1 | 10-2 | 10-3 |

CODING FORMULA

FIRST TWO DIGITS------------RESISTANCE CODE

THIRD  DIGIT-------------------MULTIPLIER CODE

EXAMPLE :    1.96K=196101=29B      12.4=12410-1=10X

#### THICK FILM CHIP RESISTORS

STANDARD E-96 VALUES AND 0603 RESISTANCE CODE

|  VALUE | CODE |  VALUE | CODE |  VALUE | CODE |  VALUE | CODE |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 100 | 01 | 178 | 25 | 316 | 49 | 562 | 73 |
| 102 | 02 | 182 | 26 | 324 | 50 | 576 | 74 |
| 105 | 03 | 187 | 27 | 332 | 51 | 590 | 75 |
| 107 | 04 | 191 | 28 | 340 | 52 | 604 | 76 |
| 110 | 05 | 196 | 29 | 348 | 53 | 619 | 77 |
| 113 | 06 | 200 | 30 | 357 | 54 | 634 | 78 |
| 115 | 07 | 205 | 31 | 365 | 55 | 649 | 79 |
| 118 | 08 | 210 | 32 | 374 | 56 | 665 | 80 |
| 121 | 09 | 215 | 33 | 383 | 57 | 681 | 81 |
| 124 | 10 | 221 | 34 | 392 | 58 | 698 | 82 |
| 127 | 11 | 226 | 35 | 402 | 59 | 715 | 83 |
| 130 | 12 | 232 | 36 | 412 | 60 | 732 | 84 |
| 133 | 13 | 237 | 37 | 422 | 61 | 750 | 85 |
| 137 | 14 | 243 | 38 | 432 | 62 | 768 | 86 |
| 140 | 15 | 249 | 39 | 442 | 63 | 787 | 87 |
| 143 | 16 | 255 | 40 | 453 | 64 | 806 | 88 |
| 147 | 17 | 261 | 41 | 464 | 65 | 825 | 89 |
| 150 | 18 | 267 | 42 | 475 | 66 | 845 | 90 |
| 154 | 19 | 274 | 43 | 487 | 67 | 866 | 91 |
| 158 | 20 | 280 | 44 | 499 | 68 | 887 | 92 |
| 162 | 21 | 287 | 45 | 511 | 69 | 909 | 93 |
| 165 | 22 | 294 | 46 | 523 | 70 | 931 | 94 |
| 169 | 23 | 301 | 47 | 536 | 71 | 953 | 95 |
| 174 | 24 | 309 | 48 | 549 | 72 | 976 | 96 |

| #### THICK FILM CHIP RESISTORS |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6. CHARACTERISTICS: |  |  |  |  |  |  |  |  |
| CHARACTERISTIC |  | #### LIMITS |  |  |  | TEST METHOD (JIS-C-5202) |  |  |
| TEMPERATURE COEFFICIENT |  | 1% | 10---100200PPM/C 101---1M100PPM/C |  |  | 5.2 NATURAL RESISTANCE CHANGE PER TEMP. DEGREE CENTIGRADE R2-R1 × 106 (PPM/℃) R1(T2-T1) R1: RESISTANCE VALUE AT ROOM TEMP. (T1) R2: RESISTANCE VALUE AT ROOM TEMP. +100℃ (T2) TEST PATTERN: ROOM TEMP., ROOM TEMP. +100℃ |  |  |
|  |  | 5% | 1---10400PPM/C 11---10M200PPM/C |  |  |  |  |  |
| SHORT-TIME OVERLOAD |  | 1% | ±(1%+0.1Ω) MAX. |  |  | 5.5 PERMANENT RESISTANCE CHANGE AFTER THE APPLICATION OF 2.5 TIMES RCWV FOR 5 SECONDS. |  |  |
|  |  | 5% | ±(2%+0.1Ω) MAX |  |  |  |  |  |
| INSULATION RESISTANCE |  | 1,000 MΩ OR MORE |  |  |  | 5.6 APPLY 500V DC BETWEEN PROTECTIVE COATING AND TERMINATION FOR 1 MINUTE, THEN MEASURE. |  |  |
| DIELECTRIC WITHSTANDING VOLTAGE |  | NO EVIDENCE OF FLASHOVER MECHANICAL DAMAGE, ARCING OR INSULATION BREAK DOWN. |  |  |  | 5.7 APPLY 500V DC BETWEEN PROTECTIVE COATING AND TERMINATION FOR 1 MINUTE |  |  |
| TERMINAL BENDING |  | ±(1%+0.05Ω) MAX |  |  |  | 6. 1.4 TWIST OF TEST BOARD: Y/X = 3/90 mm FOR 60 SECONDS |  |  |
| SOLDERING HEAT |  | RESISTANCE CHANGE RATE IS: ±(1%+0.05Ω) MAX |  |  |  | 6.4 DIP THE RESISTOR INTO A SOLDER BATH HAVING A TEMPERATUER OF 350C 5C AND HOLD IT FOR 31 SECONDS. |  |  |
| #### THICK FILM CHIP RESISTORS |  |  |  |  |  |  |  |  |
| CHARACTERISTIC | LIMITS |  |  |  | TEST METHOD (JIS-C-5202) |  |  |  |
| TEMPERATURE CYCLING | 1% |  | ±(0.5%+0.05 )MAX |  | 7.4 RESISTANCE CHANGE AFTER CONTINUOUS FIVE CYCLES FOR DUTY CYCLE SPECIFIED BELOW: |  |  |  |
|  |  |  |  |  | STEP |  | TEMPERATURE | TIME |
|  |  |  |  |  | 1 |  | -55℃ ± 3℃ | 30 MINS |
|  | 5% |  | ±(1.0%+0.05 )MAX. |  | 2 |  | ROOM TEMP. | 10 --- 15 MINS |
|  |  |  |  |  | 3 |  | +125℃ ± 3℃ | 30 MINS |
|  |  |  |  |  | 4 |  | ROOM TEMP. | 10 --- 15 MINS |
| SOLDERABILITY | 95% COVERAGE MIN. |  |  |  | 6.5 TEST TEMPERATURE OF SOLDER: 245C  5C DIPPING TIME IN SOLDER: 3 0.5 SECONDS. |  |  |  |
| HUMIDITY ( STEADY STATE ) | 1% |  | ±( 0.5%+0.1  ) MAX. |  | 7.5 TEMPORARY RESISTANCE CHANGE AFTER A 1,000 HOURS EXPOSURE IN A HUMIDITY CHAMBER. |  |  |  |
|  | 5% |  | ±( 3.0%+0.1  ) MAX. |  |  |  |  |  |
| LOAD LIFE IN HUMIDITY | RESISTANCE CHANGE RATE IS: |  |  |  | 7.9 RESISTANCE CHANGE AFTER 1,000 HOURS (1.5 HOURS “ON”,0.5 HOUR “OFF”) AT RCWV IN A HUMIDITY CHAMBER CONTROLLED AT 40℃±2℃ AND 90 TO 95% RELATIVE HUMIDITY. |  |  |  |
|  | 1% |  |  | ±(1%+0.1 ) MAX. |  |  |  |  |
|  | 5% |  |  | ±(3%+0.1 ) MAX. |  |  |  |  |
| LOAD LIFE | RESISTANCE CHANGE RATE IS: |  |  |  | 7.10 PERMANENT RESISTANCE CHANGE AFTER 1,000 HOURS OPERATING AT RCWV WITH DUTY CYCLE 1.5 HOURS “ON”, 0.5 HOUR “OFF” AT 70℃±2℃ AMBIENT. |  |  |  |
|  | 1% |  |  | ±(1%+0.1) MAX. |  |  |  |  |
|  | 5% |  |  | ±(3%+0.1) MAX. |  |  |  |  |

THICK FILM CHIP RESISTORS

7.  PACKAGING:

7.1 TAPPING DIMENSION:

UNIT: mm

TYPEA  0.2B  0.2C  0.05+ 0.1

D

- 0E  0.1F  0.05G  0.1W  0.204020.651.152.01.51.753.54.08.006031.101.902.01.51.753.54.08.008051.652.402.01.51.753.54.08.012062.003.602.01.51.753.54.08.012102.803.502.01.51.753.54.08.020102.95.62.01.51.755.54.012.025123.56.72.01.51.755.54.012.0

7.2 DIMENSION:

UNIT: mm

TYPETAPINGQUANTITYA0.5B0.5C0.5D1M2W10402PAPER10,000 PCS REEL2.013.021.060.0178100603PAPER5,000 PCS REEL2.013.021.060.0178100805PAPER5,000 PCS REEL2.013.021.060.0178101206PAPER5,000 PCS REEL2.013.021.060.0178101210PAPER5,000 PCS REEL2.013.021.060.0178102010Embossed4,000 PCS REEL2.013.021.060.017813.82512Embossed4,000 PCS REEL2.013.021.060.017813.8

PART NUMBER SYSTEM

EXPLANATION OF PART NUMBER SYSTEM

(THICK FILM CHIP RESISTORS)

ORDERING PROCEDURE (EXAMPLE: 1206 1/4W 5% 100  T/R-5000):

E

0

6

W

4

J

0

1

1

T

5

0

1

2

PRODUCT TYPE:

FILL-IN THESE 4

DIGITS WITH THE CHIP RESISTOR TYPES AS FOLLOWS:

0402

0603

0805

1206

1210

2010

2512

4D03

10P8

16P8

WATTAGE:

FILL-IN THESE 2 DIGITS WITH THE CODES AS FOLLOWS:

NORMAL SIZE

WG=1/16W

WA=1/10W

W8=1/8W

W4=1/4W

W2=1/2W

1W=1W

SMALL SIZE

SA=1/10WS

S8=1/8WS

S4=1/4WS

S3=1/3WS

07=3/4WS

SPECIAL:

WH=1/32W

RESISTANCE VALUE:

1.E-24 SERIES: THE 1st  DIGIT IS “0”,THE 2nd & 3rd DIGITS ARE FOR THE SIGNIFICANT FIGURES OF THE RESISTANCE AND THE 4th INDICATE THE NUMBERS OF ZEROS FOLLOWING;

2.E-96 SERIES: THE 1st TO 3rd DIGITS ARE FOR THE SIGNIFICANT FIGURES OF THE RESISTANCE AND THE 4th DIGIT INDICATE THE NUMBERS OF ZEROS FOLLOWING.

PACKING QUANTITY:

1=1,000PCS

2=2,000PCS

3=3,000PCS

4=4,000PCS

5=5,000PCS

C=10,000PCS

D=20,000PCS

PACKAGING TYPE:

T=T/R PACKING

B=BUIK IN POLY-BAG

C=BUIK IN CASSETTE

SPECIAL FEATURE:

0=NORMAL TYPE

E=LEAD FREE PLATING TYPE

TOLERANCE:

F=1%

G=2%

J=5%